RESISTIVE

Mohammad Hasan, Aaron Price

Wa’el Alkalbani , Mosiah Beal

Group 6

10/21/2023

Introduction and physical properties

______________________________________________________________________________

1- Cell Description

This cell consists of a NMOS and pull up resistor as a load bearing component, vs a typical Enhancement inverter. These resistive load inverters may be used in place of enhancement inverters for various reasons, including but not limited too:

- Simplicity of technology

- Higher output voltage when compared to enhancement

- Availability with older technologies

2- Cell Symbols

Figure 1: Resistive inverter (Short)

Figure 2: Resistive inverter (long)

3- Cell Truth Table (for both resistive inverters)

| Cell input | Cell output |
| --- | --- |
| A | Y |
| 0 | 1 |
| 1 | 0 |

4- Cell schematic

Figure 3: Resistive inverter (short r = 60K)

Figure 4: Resistive inverter (long R0 = r=R = 200K)

5- Cell Dimensions

Resistive

| transistor instance number | Width (nm) | Length (nm) | resistance (ohms) |
| --- | --- | --- | --- |
| Resistor (R0) | — | — | 60k |
| NMOS (M0) | 90 | 50 | – |

Resistive Long

| transistor instance number | Width (nm) | Length (nm) | resistance (ohms) |
| --- | --- | --- | --- |
| Resistor (R0) | — | — | 200k |
| NMOS (M0) | 90 | 50 | – |

6- Input X: output rise time data

Resistive

| Input rise/fall time (ps) | FO0 | FO1 | FO2 | FO4 | FO8 |
| --- | --- | --- | --- | --- | --- |
| 40 | —----- | —---- | —----- | 808.5 | —------ |

Resistive Long

| Input rise/fall time (ps) | FO0 | FO1 | FO2 | FO4 | FO8 |
| --- | --- | --- | --- | --- | --- |
| 40 | —----- | —---- | —----- | 2925.0 | —------ |

7- Input X: output fall time data

Resistive

| Input rise/fall time (ps) | FO0 | FO1 | FO2 | FO4 | FO8 |
| --- | --- | --- | --- | --- | --- |
| 40 | —----- | —---- | —----- | 118.2 | —------ |

Resistive Long

| Input rise/fall time (ps) | FO0 | FO1 | FO2 | FO4 | FO8 |
| --- | --- | --- | --- | --- | --- |
| 40 | —----- | —---- | —----- | 105.8 | —------ |

8- Data worst case Low to High propagation delay tplh

Resistive

| Input rise/fall time (ps) | FO0 | FO1 | FO2 | FO4 | FO8 |
| --- | --- | --- | --- | --- | --- |
| 40 | —----- | —---- | —----- | 590.613 | —------ |

Resistive Long

| Input rise/fall time (ps) | FO0 | FO1 | FO2 | FO4 | FO8 |
| --- | --- | --- | --- | --- | --- |
| 40 | —----- | —---- | —----- | 2082.7 | —------ |

9- Data worst case High to Low propagation delay tphl

Resistive

| Input rise/fall time (ps) | FO0 | FO1 | FO2 | FO4 | FO8 |
| --- | --- | --- | --- | --- | --- |
| 40 | —----- | —---- | —----- | 100.105 | —------ |

Resistive Long

| Input rise/fall time (ps) | FO0 | FO1 | FO2 | FO4 | FO8 |
| --- | --- | --- | --- | --- | --- |
| 40 | —----- | —---- | —----- | 90.7394 | —------ |

10- Transient analysis

Figure 5: Transient analysis on Resistive Short circuitry

Figure 6: Transient analysis on Resistive Long circuitry

Table of Transient Analysis

| TYPE | tplh (ps) | tphl (ps) | tr (ps) | tf (ps) |
| --- | --- | --- | --- | --- |
| CMOS | 275.404 | 96.995 | 391.1 | 116.6 |
| CMOS_LONG | 75.756 | 31.985 | 94.56 | 32.14 |
| Enhancement mode NFET | 323.892 | 72.1549 | 1560 | 74.74 |
| Enhancement mode NFET Long | 1034.43 | 62.3649 | 4724 | 77.4 |
| Resistive load | 590.613 | 100.105 | 880.5 | 118.2 |
| Resistive load Long | 2082.7 | 90.7394 | 2925 | 105.8 |

11- DC analysis

Figure 7: DC analysis on Resistive Short  circuitry

Figure 8: DC analysis on Resistive Long  circuitry

Table of DC Analysis

| TYPE | VIH_DC (mV) | VIL_DC (mV) | VOH_DC (mV) | VOL_DC (mV) |
| --- | --- | --- | --- | --- |
| CMOS | 613.3 | 369.374 | 1129.1 | 53.0749 |
| CMOS_LONG | 614.032 | 369.597 | 1128.76 | 53.984 |
| Enhancement mode NFET | 344.75 | 624.697 | 508.485 | 117.365 |
| Enhancement mode NFET Long | 270.272 | 550.588 | 528.809 | 66.047 |
| Resistive load | 677.123 | 307.049 | 1126.49 | 115.919 |
| Resistive load Long | 558.172 | 239.854 | 1132.73 | 56.047 |